| STAR5000 Patient Monitor |  |  |
| --- | --- | --- |
| Safety Standards |  |  |
| ISO 13485:2003 approved, CE marking according to MDD93/42/EEC |  |  |
| Physical Characteristics |  |  |
| Product Size: | 420㎜×250mm×340mm |  |
| Net Weight: | 5.0kg |  |
| Display: | 12.1’’ Color TFT LCD touch screen |  |
| Resolution: | 800X600 |  |
| Method: | folded |  |
| Operation Environment |  |  |
| Temperature: |  | 0-40℃ |
| Humidity: |  | 15-85% |
| Power Supply |  | AC100-250V, 50/60Hz |
| Battery Type: |  | Rechargeable lithium-ion battery |
| Battery Capacity: |  |  |
| Battery Recharging Time: |  | Maximum 6.5 hours for charging; |
| Battery working period: |  | 2 hours for continuous working |
| Interface: |  | USB(data input or output, upgrade) AC power supply socket Parameter cable interface RS232 port RJ45 port |
| Data: |  | Input/output(patient’s case) |
| Network: |  | Central Monitor System |
| Performance Specifications |  |  |
| Printing |  |  |
| Recording parper: |  | 112mm*90mm*150 Z-fold, thermo sensitive |
| Fast print speed: |  | 25mm/s(stored traces) |
| Paper Advance Speed: |  | 1/2/3cm/min selectable |
| Record Message: |  | Data,t ime, TOCO type,paper speed,FHR type,bed no.,etc. |
| Printer: |  | Built-in thermal recorder; external laser printer |
| Ultrasound |  |  |
| Technique: |  | Ultrasound Pluse Doppler with autocorrelation |
| Pulse repetition rate: |  | 2 KHz |
| Ultrasound: |  | 1.0MHz ±10% |
| FHR Range: |  | 30bpm- 240bpm |
| Resolution: |  | 1bpm |
| Accuracy: |  | ± 2 bpm |
| TOCO |  |  |
| TOCO range: |  | 0-100 Relative(%) |
| Resolution: |  | 1% |
| Zero Mode: |  | Manual |
| Non-linear error: |  | ≤±10% |
| Fetal Movement(FM) |  |  |
| Mode: |  | Auto; manual |
| Technique: |  | Pulsed Doppler Ultrasound |
| Range: |  | 0-100(%) |
| Resolution: |  | 1% |
| Fetal Stimulator |  |  |
| Method: |  | Vibration |
| Mode: |  | Manual(3 times), continuing |
| Voltage: |  | DC 3V, (AA battery) |

Standard Configuration :

Single FHR,TOCO,FM

Optional:

Twins , fetal stimulator